Installation Instruction for Transdaptor 13

For 21” whaler Transom Extension brackets

From 1959-1972 Models

Step 1:		Clean Transom with Acetone inside and out

Step 2:		Remove Ski Eyes if needed

Step 3:	On inside of Transom, measure Transom and mark center with pencil (to ensure proper alignment)

Step 4:		Silicone Bracket on contact area

Step 5:	On inside of Transom bracket, there is a center punch dot-align sot with center of Transom

Step 6:	Using a soft hammer, tap bracket down evenly, and making sure center is aligned.

Step 7:	Drill thru transom and inner plate with 1 1/32 drill bit

Step 8:	Silicone around stainless steel bolts and install

Step 9:	Clean up any excess silicone with acetone and rags

Step 10:	Silicone around edges and gaps on top

Step 11:	Reinstall Ski Eyes if needed